Analysis of Brownfields Cleanup Alternatives (ABCA)

Lost City School – Hulbert, Oklahoma

Prepared by: Cherokee Nation Environmental Programs Brownfields Program

- Introduction & Background

a). Site Location: The Lost City School Property is located at 13243 N. Lost City Road, Hulbert, Cherokee County, Oklahoma.

b). Purpose: This Analysis of Brownfields Cleanup Alternatives (ABCA) is intended as a tool to ensure and document the appropriate type of cleanup is selected to address environmental contamination issues at the Lost City School. The preferred remedial action considers the site characteristics, surrounding the environment, land-use restrictions, potential future uses, and cleanup goals.

c). Previous Site Use(s): The Subject property is the former site of the Lost City School, which consists of 35.6 acres with three school buildings, one gymnasium, one shop building, two small buildings and two baseball fields with include two backstops, two dugouts and a concession building. There is one capped well located on the property.

The exact date of the school is unknown but estimated to be 1901 or when the original schoolhouse was constructed of rock and then enlarge beyond the one room school by cinder block additions and then metal building extensions in the later years. The school has a significant Cherokee History, and Cherokee Nation used the school as Cherokee Language for their immersion program for children and adults could learn from Cherokee Language classes. The Tribe attempted to save the building but the buildings were in a bad state of deterioration and has become a health hazard. The Property has not been in use since 2008 when the Lost City School closed.  Cherokee Nation purchased the property in 2025.

c). Site Assessment Findings:

- Phase I ESA noted the potential environmental risks such as asbestos, lead-based paint (LBP), and mold based on the age of the buildings and visual inspection.

d). Project Goal: The property is to be cleaned up for reuse as part of the Cherokee Nation Community Development initiative. The Tribe would like to preserve the land for historical and cultural purposes. There has been discussion about cleaning up the property and creating a residential housing community, with a community building, playground and a storm shelter for the community to use.

e). Regional and Site Vulnerabilities: The Lost City School Property is located in the Lost City Community, with the physical address being 13243 N. Lost City Road, Hulbert, Oklahoma. The regional area and the site has cultural significance as one of the communities established as one of the areas that Cherokees used after the removal. Currently the site is in a rural setting, located just north of Hulbert, Oklahoma.

II. Applicable Regulations and Cleanup Standards

- Cleanup Oversight: The project will be managed by Cherokee Nation Management Resources and Cherokee Nation Brownfields Program.  A work plan will be created by the selected vendor, for removal operations to be carried out, the work plan will outline safe work procedures and clearance guidelines.

- Cleanup Standards: Cleanup will comply with EPA and state standards for asbestos and lead, with specific consideration for the planned future use of the property.

III. Evaluation of Cleanup Alternatives

Cleanup Alternatives Considered:

1. No Action: No remediation would be conducted.

2. Asbestos, lead-based paint, and mold removal with demolition: removal of asbestos, lead-based paint, and mold with complete demolition.

Effectiveness, Implementability, and Cost:

- Alternative 1: No Action:

- Effectiveness: does not address contamination; exposure remains.

- Cost: None.

- Implementability: Easy, but not effective.

- Alternative 2: Full Site Cleanup and Removal of Contaminants and Replacement.

- Effectiveness: Total demolition completely removes the risk of exposure by eliminating all contaminants and future contaminants.

- Implementability: Moderate difficulty due to the need for site preparation, demolition, removal and disposal of hazardous materials.  Ensures that the historic property will prevent future contaminants.

Recommended Cleanup Alternative:

- Alternative 2: Full Site Demolition, Cleanup and Removal is advised since it effectively eliminates all pollution and exposure threats. The property is abandoned and not suitable for reuse in it’s current state.  Alternative 2 is the best course of action for advancing the project. This alternative can guarantee that cleanup can be accomplished and make the property suitable for reuse and redevelopment.

IV. Conclusion

The recommended cleanup alternative is to proceed with the Alternate 2: full site demolition and site cleanup to remove the asbestos, lead–based paint and building materials to ensure that the site is safe for future reuse.