Analysis of Brownfields Cleanup Alternatives (ABCA)

Stucky Hall – Markoma Property, Tahlequah, Oklahoma

Prepared by: Cherokee Nation Environmental Programs Brownfields Program

- Introduction & Background

a). Site Location: The Stucky Building is located on the Cherokee Nation Markoma Campus at 886 Markoma Dr., Tahlequah, Cherokee County, Oklahoma.

b). Purpose: This Analysis of Brownfields Cleanup Alternatives (ABCA) is intended as a tool to ensure and document the appropriate type of cleanup is selected to address the environmental contamination issues at the Stucky Hall site. The preferred remedial action considers the site characteristics, surrounding environment, land-use restrictions, potential future uses, and cleanup goals.

c). Previous Site Use(s): The Subject Property was the site of the original Cherokee Nation Male Seminary, which opened in 1851 and burned down in 1910.  Between 1910 and 1951, after Cherokee Nation sold the property, the Subject Property was used primarily as a private residence and for agricultural purposes.  In 1961, Markoma Bible Academy opened and operated until 2002. During that time, several buildings were constructed. Stucky Hall (the Subject Property) served as the cafeteria, office space and classrooms.  Cherokee Nation was able to acquire the land back and has used several of the property buildings for Cherokee Nation Offices.  Cherokee Nation has remolded the Markoma Gym, Girls Dorm, Boys Dorm and the Library.  Cherokee Nation was never able to use Stucky Hall, due to asbestos, mold, and structure issues.

c). Site Assessment Findings:

- Phase I ESA noted the potential environmental risks such as asbestos, lead-based paint (LBP), and mold based on the age of the buildings.

- Phase II ESA Findings: Asbestos  Category 1 non-friable ACM was identified. Stucky Hall has approximately 1,500 square feet of dark brown 9X9 vinyl floor tiles, green 9x9 vinyl floor tiles, and tan 9x9 vinyl floor tiles located throughout the upstairs area offices and hallways.  Regulated Asbestos Containing Materials (RACM) were identified; approximately 6,300 square feet of asbestos containing fireproofing on the concrete beam and joist ceilings are located throughout the 1st and 2nd floor areas.  The asbestos is also crumbling and in a dust producing state.  d). Project Goal: The property is to be cleaned up for reuse as part of the Markoma Campus, with focus on addressing environmental risks for future renovations or redevelopment.

e). Regional and Site Vulnerabilities: Stucky Hall is located on the Markoma Campus, where there are office spaces in different locations throughout the property. There is a walking trail from the Markoma Fitness Center and a paved road goes by Stucky Hall that is used by Cherokee Nation Tribal Citizens, Non-tribal residents and Cherokee Nation Employees. The property has a high cultural significance and has a sacred well/stream nearby.

II. Applicable Regulations and Cleanup Standards

- Cleanup Oversight: The project will be managed by Cherokee Nation Management Resources and Cherokee Nation Brownfields Program.  A work plan will be created by the selected vendor for removal operations to be carried out; the work plan will outline safe work procedures and clearance guidelines.

- Cleanup Standards: Cleanup will comply with EPA and state standards for asbestos and lead, and removal of the mercury switches in the thermostats, with specific consideration for the planned future use of the property.

III. Evaluation of Cleanup Alternatives

Cleanup Alternatives Considered:

1. No Action: No remediation would be conducted.

2. Asbestos, Mold: Abatement of asbestos – containing floor tiles and mastic in identified areas.  Safe removal of lead-based paint from affected areas.  Removal of moisture intrusion and mold growth in the Stucky Hall.  Replacement of the roof and sealing and fixing structure openings, replace drywall for the mold damaged areas, windows and door frames.

3. Asbestos and mold Removal with demolition: Removal of asbestos and mold with complete demolition.

Effectiveness, Implementability, and Cost:

- Alternative 1: No Action:

- Effectiveness: does not address contamination; exposure remains.

- Cost: None.

- Implementability: Easy, but not effective.

- Alternative 2: Asbestos and Mold Removal:

- Effective in eliminating exposure risks.

- Cost: Unknown (for all remediation).

- Implementability: Requires coordination with contractors and adherence to regulations. Present contamination will be gone but it does not address the structural issues or safety uses of reusing the building.

- Alternative 3: Full Site Cleanup and Removal of Contaminants and Replacement:

- Effectiveness: Total demolition completely removes the risk of exposure by eliminating all contaminants and future contaminants.

- Cost: Unknown (for demolition and cleanup). Will be bid out.

- Implementability: Moderate difficulty due to the need for site preparation, demolition, removal and disposal of hazardous materials. Ensures that the historic property will prevent future contaminants.

Recommended Cleanup Alternative:

- Alternative 3: Full Site Demolition, Cleanup and Removal is advised since it effectively eliminates all pollution and exposure threats. Stucky Hall served as cafeteria, office spaces and classrooms for the former Markoma Bible Academy. The property is abandoned and not suitable for reuse. Alternative 3 is the best course of action for advancing the project. This alternative guarantees that cleanup can be accomplished and make the property suitable for reuse and redevelopment.

IV. Conclusion

The recommended cleanup alternative is to proceed with the Alternative 3: full site demolition and site cleanup to remove the asbestos, lead – based paint and building materials to ensure that the site is safe for future reuse.